Respuesta del tejido conectivo de ratas al endo-cpm sealer

Canzani Jorge ,Salaverry Graciela* ,Mandalunis Patricia, Bidegorri Marcela

Carrera de Especialización en Endodoncia USAL/AOA

Cátedra de Histología y Embriología de FOUBA

Bioterio Central de FFy BUBA

Resumen

Objetivos: Se evaluó  histológicamente la reacción inflamatoria del tejido conectivo de ratas  producida por un nuevo sellador endodóntico, el Endo-C.P.M.sealer, (EGEO- Argentina) ,a base de trióxidos minerales agregados y se la comparó  con  Cemento de Grossman, ( Tedekim Argentina ) a base de óxido de zinc eugenol, llevados en  tubos de silicona e implantados en  tejido celular subcutáneo de ratas. Para el control negativo se  utilizaron tubos vacíos .

40 ratas Wistar hembras de 150 a 200 g, fueron sacrificadas en grupos de 10 a los  7, 21, 30 y 60 días.

Se determinó  los  tipos celulares presentes  en las áreas reactivas inflamatorias y las  calcificaciones distróficas bajo microscopía óptica.

Los resultados  demostraron que el área de reacción inflamatoria en los tubos obturados con Endo-C.P.M. fue mayor que con  Grossman y no mostró resolución a los 60 días. Presencia de calcificaciones distróficas fueron observadas con el Endo-CPM sealer.

Bibliografía: 1.Rickert UG,Dixon CM. J Amer Dent Assoc 1938;25:178

2.Endodontic Practice Lea y Febiger. Philadelphia _USA

3.Eráusquin, J.;Muruzábal M.;(1967): A method for root canal treatment in the molar of the rat.Oral Surg.Oral Med. Oral Path.24(4):540-6

4.Eráusquin J.;Muruzábal M.;1967:Root canal fillings with zinc oxide-eugenol cement in the rat molar.Oral Surg. Oral Med.Oral Path.24(4):547-558

5.Eráusquin J,Muruzábal M.,1968:Tissue reaction to root canal cements in the rat molar Oral Surg.Oral Med.Oral Path 26(·):360-373

6.Friend L A.,Browne R M. Tissue reactions to same root filling materials implanted of  the bone of rabbits.Archs Oral Biol.1969;14:629-638

7.Jin –Su Kim, Seung –Ho Baek,Kwang-Sihik Bae.In vivo study on biocompatibility of newly Developer calcium phosphate root canal sealers. J Endod 2004;30(10):708-11

8.Kaplan A., Ormaechea MF, Pirca M, Canzobre MC, Ubios MA. Rheological         Properties and biocompatibility of endodontic sealers. Int Endod J. 2003;36(8):527-32

9.Zmener O.Tissue response to a new methacrylate –based root canal sealer:preliminary observations in the subcutaneous connective tissue of rats

J Endod 2004; 30(5):348-351

10. Tagger M,Tagger E,1986 Subcutaneous  reactions to implantation of tubes with AH26 and Grossman sealer.Oral Surg. Oral Med. Oral Pathol 1986; 62 :434-40

11.Bezerra Silva,Leonardo M.,Faccioli L,Figueredo F.Inflamatory response to calcium hydroxide based root canal sealers J Endod 1997;23 (2):86-90

12.Berit Olsson,Anthony Sliwkoski,Kaare Langeland.Intraosseuous implantation for biological evaluation of endodontic materials J Endod 1981;7(6):253-265

13.Jacob Saidon,Jianing He,Qiang Zhu,Kamram Safari, Lanz Spangberg.Cell and tissue ractions to mineral trioxide aggregate and Pórtland cement.Oral Surg.Oral Med. Oral Pathol. 2003; 29(4):483-489.

14.Holland R, de Souza V,Nery MJ Reaction of rat conective tissue to implanted dentin tubes filled  with mineral trioxide aggregate. Portland cement or calcium hydroxide.Brazilian Dental J 2001;12:3-8

15.Holland R,de Souza V,Nery MJ. et al Reaction with a white mineral trioxide aggregate.Brazilian Dental J  2002;13:23-6

16.Federation Dentaire Internationale, Recommenden Standard practices for biological evaluation of dental materials.Part 4.11:subcutaneousimplantation test. Int Dent J.1980; 30:173-4